# Instructions

### Start MatLab

(below, are instructions are for MatLab R2024b, but may be applicable to other versions)

In HOME tab, Layout pulldown menu, make sure that you can see the windows: Current Folder, Workspace, Panel Titles, Toolstrip panels:

### Tell Matlab where EasySpin is

Matlab should know where the program that runs your *.m file is. For this, go to Set Path in the HOME menu and navigate (Add Folder…) to the EasySpin location, which is, in the case of my laptop, C:\Users\svist\Box\myBox\My_Matlab_stuff\easyspin-6.0.6\easyspin; Then click Save and Close. Sometimes Matlab does not keep this information in later starts of the program. If this is the case, do it every time you start simulating EPR spectra while using EasySpin.

### Set up environment for specific simulation

Set a folder where all simulations will be run and name it. Paste the three *.m files to that folder,

Also paste Experimental.txt file to the same folder. You might also paste there the Excel file Compare different simulationsl.xlsx.

### Setting Current Folder and opening the three *.m files

In Current Folder, navigate to the folder with the three *.m files. The output will be generated in this folder. Alternatively, you can open your *.m files in MatLab from elsewhere, say by clicking on them in Windows Explorer. When you try to run a file, Matlab will suggest switching Current Folder to the location of your file.

Once you open the *.m files, they will appear as tabs in the Editor panel:

TRSSA_Y_simulate.m  (this is the file to run individual simulations when C1 and  could be altered as well as several other parameters),

TRSSA_Y_fit.m (this is the file to run best fit; it works together with the

TRSSA_Helper.m file which has to be in the same directory, but does not have to be open; still it is a good practice to see what is inside the file).

If you now run the TRSSA_Y_simulate.m file (from EDITOR tab) it should generate a simulated spectrum and display it together with the experimental spectrum specified as Experimental.txt

### Setting YOUR OWN experimental spectrum

You should re-define the Experimental.txt file in the attached demo (defined on 1024 datapoints), in accordance with the experimental spectrum you are going to simulate. Save your own experimental spectrum as a txt file in the same format as the demo Experimental.txt which you found in the ZIP from me.

### Setting Instrumental parameters in TRSSA_Y_simulate.m

% Instrumental parameters ------------------------------------------------

Set experimental parameters: MW frequency, field range in mT and data points number. (MW frequency might be found in the original paper reporting the spectrum; This value, Exp.mwFreq, should also be specified in TRSSA_Y_fit.m files).

Exp.mwFreq = 9.474;              % MW frequency, GHz

Exp.Range = [333.68 342.13];     % Field range, mT

Exp.nPoints = 1024;              % Number of data points

### Setting Simulation parameters in TRSSA_Y_simulate.m

% Simulation parameters --------------------------------------------------

roC1= 0.41; 	% spin density on C1, must be between 0.35 and 0.42

theta= 43.0 	% theta, degrees; should be between -90 and +90 degrees

abp= 118; 	% "angle between protons", degrees - angle from the blue methylen proton to the pink one, anti-clockwise

Am= 1; 	% modulation amplitude, Gauss

cAm= 0; 	% positive or negative correction value to Am, Gauss

The roC1 and theta are the key parameters of TRSSA, C1 and θ. The abp should be close to 120 degrees (DFT often shows it is 118o). The sum of Am and cAm defines the individual line width, both values are in Gauss. The value of Am is sometimes reported in the original paper; for the simulation it is the value of (Am+cAm) that matters.

### Run a simulation in TRSSA_Y_simulate.m

To make experimental and simulated spectra similar in intensity, you can adjust scaling factor:

ScaleExp = 0.5; 		% Scaling factor for experimental spectrum when plotting the two together

In the % Simulation parameters -------------------------------------

start varying roC1 and theta values to roughly approximate the experimental spectrum. You can then vary, but slightly, other simulation parameters:

abp =

Am =

cAm =

Bprime =

After a simulation, in the Workspace panel, double-click on the variable AA_SimSpec (it should be at the top of the list). In the opened Variable Editor, select the two columns, copy and paste into Excel file Compare different simulations.xlsx.  You will be able then to compare different individual (manual) simulations. Keep pasting different simulations (in the two column format) to the Excel spreadsheet. Any of the simulations, when copy-pasted to columns F-G (between the two vertical red lines), will display the experimental/simulated spectra in the figure. It is a good practice to paste a set of different simulations to F-G, one after another, and then click Back button in Excel as many times to see how the simulated spectrum changes.

### Fitting mode (the case of four parameters to vary)

In the TRSSA_Y_fit.m file, specify Instrumental parameters, make sure they are the same as in your TRSSA_Y_simulate.m file:

% Instrumental parameters ------------------------------------------------

Exp.mwFreq = 9.474;              % MW frequency, GHz

Exp.Range = [333.68 342.13];     % Field range, mT

Exp.nPoints = 1024;              % Number of data points

Another parameter which you should set the same in the two files is Am under:

% Simulation parameters in TRSSA_Y_simulate.m and under

% Define static parameters in TRSSA_Y_fit.m

Now, under % Parameter ranges for fitting --------------------------------

specify the starting values and the ranges of variation of the parameters the software will be varying. In the demo file in the ZIP, there are FOUR parameters to vary:
roC1   theta    abp     cAm

Make sure that you start from the values you already have manual simulations for, and the ranges of variations are not large.

% roC1   theta    abp     cAm

% p(1)    p(2)    p(3)    p(4)

start = [0.41    43      119     0.0];

lower = [0.39    40      118    -0.9];

upper = [0.42    46      120     1.5];

Run TRSSA_Y_fit.m file.

A new window appears showing the experimental spectrum and the parameters to vary.

Click Start fitting button at the top-right.

Optimising four parameters in the esfit routine might take several minutes. (~10 min for 4 parameters to be varies within small ranges; the time might go to ~40 min if the ranges are only slightly increased). However, you can set the TRSSA_Y_fit.m file and the TRSSA_Helper.m to vary the parameters individually or in smaller groups.

### How to fit while varying fewer parameters

Both TRSSA_Y_fit.m and TRSSA_Helper.m have to be edited.

Suppose we would like to run a fit when rC1 and theta are varied, but both abp and cAm stay constant.

In TRSSA_Y_fit.m, you have to remove the starting point and variation ranges for abp and cAm:

% roC1    theta     abp     cAm

% p(1)    p(2)      p(3)    p(4)

start = [0.41    43  ];  % 119     0.0];

lower = [0.39    40  ];  % 118    -0.9];

upper = [0.42    46  ];  % 120     1.5];

In TRSSA_Helper.m, instead of

abp =      p(3); % "angle between protons"

cAm =      p(4);

we should silence the two variables and explicitly specify their values:

% abp =      p(3); % "angle between protons"

% cAm =      p(4);

abp = 118;

cAm = 0.999;

Once we run now a fit, it will take only ~3.5 min, even with good ranges of the two parameters (because the other two are fixed).

After the fitting is finished and the Log box (bottom-right) reports a Goodness of fit (see below the NB comment), simulate the spectrum again in TRSSA_Y_simulate.m using the found best-fit parameters. Transfer the simulated spectrum to Excel and see if it is better than before.

NB. In Goodness of fit:

ssr             3.17318e+06  The Sum of Squares due to Regression (SSR) or Explained Sum of Squares (ESS) is the sum of the squares of differences between the predicted and experimental values, over 1024 datapoints.

rmsd            55.667  Root Mean Square Deviation, also known as RMSE (Root Mean Square Error), is a frequently used measure of the differences between values predicted by a model or estimator and the values actually observed. It is, in our case, (ssr/1024)^(1/2), or (3173180/1024)^(1/2)=55.6669

That’s it!

Dima Svistunenko

svist@essex.ac.uk